Technical Clarification

| No. | Adish Comment | Sureway 1ST Reply | Adish Reply | Meeting (dated 17/Mar/2025) Adish Attendance: Mr. Mataji , Shahani and Pourkarim. SUREWAY Attendance: Mr. George Han |
| --- | --- | --- | --- | --- |
|  | Refer to 3.2. of Technical Proposal rev 02: Sodium Sulfide. | Sodium sulfate is input error Sodium sulfite also is input error. Corrected production name is Sodium sulfide. | Noted. | - |
|  | Refer to 3.2. of Technical Proposal rev 02: Please clarify the number of H2O in Na2S formula as product. And also clarify about production of Sodium sulfide in two cases: Na2S + 3H2O Na2S + 5 H2O | Industrial products typically exist in the form of decahydrate (Na₂S·10H₂O), but in actual production, the amount of crystal water may vary between 0 and 6 due to process differences. Industrial products typically exist in the form of decahydrate (Na₂S•10H₂O), but in actual production, the amount of crystal water may vary between 0 and 6 due to process differences. According to the 60% purity of sodium sulfide, 60% of sodium sulfide is a mixture of Na2S • 2H2O, Na2S • 3H2O, and Na2S • 5H2O | The amount of crystal water could be adjusted by process variables, please consider it in design and operation manual. | Min 60% purity is possible which is produced in evaporation section. |
|  | Refer to 3.3. of Technical Proposal rev 02: As per MR the sodium sulfite is 0.5 wt% (max) and Sodium thiosulfate is 0.8 wt% (max) Sodium carbonate is 0.5 wt% (max). | Can meet the requirement of : Sodium sulfite ≤ 05% WT, Sodium thiosulfate ≤ 0.8 wt% Sodium carbonate ≤ 0.5 wt% | Noted. | - |
|  | Sodium Hydrosulfide Specifications as product to be included. | Will provide in update proposal | Noted. Will check the revised proposal. | Check the revised technical proposal. The production of Na2S and NaHS is flexible to switch based on market demand, also between liquid and solid. The facilities for production of Na2S and NaHS is identical, with only the operation conditions requiring adjustment. |
|  | There is a spent caustic stream from LPG treatment unit, please check during detailed design to receive this stream for production of Na2S. For information, in LPG treatment Unit (LPT), there is pretreatment of LPG with caustic to remove H2S content of LPG and a spent caustic will be produced accordingly. | All liquid used for Sodium sulfide production, without waste liquid discharge, we will update the PFD and general consider the process during PDP design. |  | During the meeting, Adish explain about the existing facilities in LPG Treatment Unit and spent caustic solution production. A pretreatment facility removes H2S from LPG stream using caustic solution (NaOH). As H2S is absorbed by the caustic solution, it reacts to form spent caustic solution (sodium hydrosulfide (NaHS) or sodium sulfide (Na2S)), depending on the NaOh concentration. SUREWAY is requested to review the spent caustic stream for receipt at B.L. of SSP Unit for process and production purpose. It is agreed that Adish will provide information about LPG stream specifications before and after caustic treatment for SUREWAY review and evaluation. Additionally, the available caustic at B.L. of SSP unit is 50 wt% Caustic solution. (For more information about the caustic system refer to SACR-DE-CDS-PR-DBA-0501.) SUREWAY will review and provide feedback accordingly. |
|  | Refer to 4.1. of Technical Proposal rev 02: Please check and evaluate the possibility of hydrocarbon condensation (heavy content of feed gas) in absorber tower during design stage. | OK. Detail specification shall be provided during PDP design. Material balance and energy balance shall be provided during PDP design. | Noted. | - |
|  | Refer to 4.1.2. of Technical Proposal rev 02: Please clarify how the concentration of NaHS and Na2S will be adjusted. | Please confirm the production capacity of NaHS and Na2S. |  | Refer to the revised technical proposal. |
|  | Please clarify about the recovery of evaporated water and its rate. | ≥95% | The specification and usage of the recovered water? | SUREWAY explain that the impurity in the products originate from water or Caustic soda solution. The recovered water will be used in the production process. |
|  | Refer to 5.2. of Technical Proposal rev 02: The available utility conditions are specified in "BASIC ENGINEERING DESIGN DATA (SACR-DE-GEN-PR-SPC-0004-06)", section 9. UTILITIES CHARACTERISTICS" Please consider the utility characteristics during developing Basic Design. | OK, We will follow this instruction to update the technical proposal | Noted. | - |
|  | Refer to 5.2. of Technical Proposal rev 02: please clarify about "Tap Water" and check the available water specification in the plant based on "BASIC ENGINEERING DESIGN DATA (SACR-DE-GEN-PR-SPC-0004-06)". Note: The Air Cooled shall be maximized as much as possible technically and economically. | Ok, We will follow this instruction to update the technical proposal |  | Tap water is fresh water that serves as makeup water in SSP Unit. |
|  | Refer to 2.1. (Design data) of Technical Proposal rev 02: Please check and include the following documents in scope of "Basic Design": Design Basis and Design Criteria Process Description PFD and HMB Material Selection Diagram PID Utility Summary Chemical List Waste and Effluents List Equipment List Equipment Data Sheet Equipment Drawing Cause & Effect Safety Diagram Control Philosophy Process Instrument Data Sheet Safety and Control Valve Process Data sheet Analytical procedures Safety Procedure HAZID / HAZOP / SIL Study plant general and Equipment Layout 3D model (30%) PDMS file Electrical Load List Electrical Single Line I/O List Operation Manual Note1: The native file shall be submitted Note 2: The calculation to be developed and submitted. Note 3: The applicable standards, criteria and specification for basic design shall be specified. Note 4: Consider P&ID Review meeting and 3D model review meeting. Note 5: Please find the project procedure documents as attached files for consideration during design. | Ok, We will follow this instruction to update the technical proposal | Noted. | Check revised Technical Proposal. |
|  | The feed and product conditions are specified as per “REQUISITION FOR DESIGN OF SODIUM SULFIDE PRODUCTION UNIT” (SACR-DE-GEN-MA-MRQ-9001) as attached file, please check an confirm. | Noted, will double check it and proposal | Noted. | Check revised Technical Proposal. |
|  | As per “REQUISITION FOR DESIGN OF SODIUM SULFIDE PRODUCTION UNIT” (SACR-DE-GEN-MA-MRQ-9001) , Na2S and NaHS as defined as products (solution and flak). A flexibility in the production of two key products, NaHS and Na2S (Flake or solution), into design is required. This flexibility will be crucial for adapting to market demands and optimizing operational efficiency. | Noted, Please confirm the production capacity of NaHS and Na2S. | Refer to Item C7. | Check revised Technical Proposal. |
|  | Please confirm the Na2S and NaHS specification as per “REQUISITION FOR DESIGN OF SODIUM SULFIDE PRODUCTION UNIT” (SACR-DE-GEN-MA-MRQ-9001) , section 7. | Noted, Please confirm the production capacity of NaHS and Na2S. |  | Check revised Technical Proposal. |
|  | Please confirm the maximum H2S content of treated gas will be 100 ppm vol. | The H2S content in treated gas can be reduce to less than 100ppm (Vol), but need optimize treatment process. | Noted. | - |
|  | The design criteria and applicable standard list to be defined. |  |  | As this unit is part of refinery, please follow the applicable code and standards (SRIC-BE-GEN-PM-SPC-0001-04). |
|  |  |  |  | SURWAY explained the reference list of sulfur chemical plants. It was noted that only one reference pertains to the production of sodium sulfide from H₂S, which is currently under construction and is expected to become operational this year. SURWAY will provide more detailed information regarding this reference and SUREWAY’s scope of work in this project. |